Q procedure

- Import LODES data from R

- Import ferry landings (delete non-NYC Ferry landings manually: Whitehall, Governor’s Island, and Battery Maritime)

- Import subway stations

- Import boro boundaries

Clip LODES to boro boundaries

Calculate 0.5 mi buffer, dissolved around subway stations, clip to boro boundaries, calculate area

- 129.365 sq mi within 0.5 mi of subway stations.

Calculate isochrones, 15 min walk from ferry landings, dissolve, calculate area

- 10.844 sq mi within walking distance from ferry landings

Calculate area within 15min walk from ferry landings NOT covered by subway

- Difference: input 15min_walk, overlay subway_buffer

- 3.649 sq mi within walking distance from ferry landings NOT within 0.5 mi of subway

Identify census tracts that have areas within 15 mins walk from ferry

- Select by location census_lodes_clipped: intersect with 15min_walk_ferry, export as a new layer

  - 405,301 resident workers

  - 985,695 jobs

  - 878,451 overall population

Identify census tracts that have areas within 15 mins walk from ferry NOT covered by subway

- Select by location census_lodes_clipped: intersect with difference_15min_walk_ferry, export as a new layer

  - 189,761 resident workers

  - 376,755 jobs

  - 435,873 overall population

Identify census tracts that have areas covered by subway

- Select by location census_lodes_clipped: intersect with subway_buffer, export as a new layer

  - 2,811,393 resident workers

  - 3,492,441 jobs

  - 6,954,163 overall population

Identify LODES statistics on a landing-by-landing basis

- Calculate 15-minute walking isochrones for ferry landings.

- Join by location census_lodes_clipped: intersect to landings_isochrones.

- Concatenate landings_isochrones: group by isochrone ID, sum census data.

- Manually rename isochrones to match ferry landing names.

Identify the subway stations that are within 15 mins of ferry landings

- Select By Location: vector research tools  select by location

- Select features from: subway_stations ; where the features intersect ; by comparing to the features from: 15min_walk_ferry; create new selection.

- Export selection as new shapefile layer: subway_stations_15min_walk_ferry

- Export selection as new csv to use to clean subway ridership data: subway_stations_15min_walk_ferry.csv

Import cleaned [in R] Ferry Ridership data into QGIS

- Layer  add layer  add delimited text layer

Use Field Calculator to rename ferry landing stop names to match the ridership data

Join ferry ridership to ferry landings

Calculate AM to PM ratio

- Field Calculator  Create new decimal field  hourly_am / hourly_pm

- Save as duplicate layer for easy symbolization

# Note:

The subway ridership dataset included in this repository is cleaned to significantly reduce size.

R code to clean MTA Subway Hourly Ridership: 2020-2024, which was downloaded after the data was queried on data.ny.gov: filtered to 2022, subway only, removed unnecessary columns: transit_mode, payment_method, fare_class_category.

# Clean subway ridership data

MTA_Subway_Hourly_Ridership_2022 <- read_csv("r_data/MTA_Subway_Hourly_Ridership__2022.csv") %>%

clean_names()

#Import QGIS output: subway stations within 15 mins of ferry landings

subway_stations_15min_walk_ferry <- read_csv("q_output/subway_stations_15min_walk_ferry/subway_stations_15min_walk_ferry.csv") %>%

clean_names()

# Filter ridership to ONLY stations within 15-min walk of ferries

subway_ridership_15min_walk_ferry <- MTA_Subway_Hourly_Ridership_2022 %>%

semi_join(

subway_stations_15min_walk_ferry,

by = c("station_complex_id" = "complex_id")

)

#Save as CSV

write_csv(subway_ridership_15min_walk_ferry, "r_output/subway_ridership_15min_walk_ferry_2022.csv")